# CSE 320-04 Quiz 5 (Set B)

Marks - 15	Time - 25 min

| Name |  |
| --- | --- |
| ID |  |

- Consider the following set of codewords: 	[1.5 + 1.5  = 3]

{10010, 01011, 11111, 01001}

- How many bits of error detection can we get if we use these codewords?

- Find how many bits of error correction we can get.

- Consider the cyclic redundancy check for the following case: [4 + 3 = 7]

Dataword: 10011

Divisor: 1011

  - Find the codeword calculated by the sender.

  - Show the verification process by the receiver when the 0-th and 3-rd bits of the codeword are flipped when receiving.

- You are given a set of data: {0101, 1010, 1110, 1101}	[3 + 2 = 5]

  - Find the checksum calculated by the sender.

  - Find the verification process of the receiver when the  0-th bit of the checksum is flipped.