# CSE 320

### Quiz 2	Group 12

### Full Marks: 15	Time: 15 min

| Name |  |
| --- | --- |
| ID |  |

| 1 | Given a signal in the time domain, express it in the frequency domain: x(t) = 5cos(3πt+6)+4sin(4πt+9) | 5 |
| --- | --- | --- |
| 2 | A signal with 200 milliwatts power passes through 10 devices, each with an average noise of 2 microwatts. What is the SNR? What is the SNRdB? | 5 |
| 3 | A line has a signal-to-noise ratio of 1000 and a bandwidth of 4000 KHz. What is the maximum data rate supported by this line? | 5 |