Assignment 2

Chapter 2

Note: For the questions below, if necessary, Consider 5 digits while performing the decimal to binary conversion of the floating portion.

## Question - 1:

Given a machine code, what do we understand by looking at the Opcode field?

## Question - 2:

Suppose you have converted LD X9, 10(X21) to its equivalent RISC-V machine code. Now based on which field machine will understand the size of the data being loaded into register X9?

## Question - 3:

What is a Program Counter? Explain the importance of a Program Counter with example(s).

## Question - 4:

if ( A[3] != A[6]){

if (A[3] == 0) {

A[3] = A[3] + 2;

}else{

A[6] = A[6] / 16;

}

}else{

A[6] = A[6] * 8

}

a) Construct the equivalent RISC-V code of the above mentioned C code.

Base addresses of array A and B are in register  X20 and X21; Consider both arrays as double word

b) From your converted codes, pick any instruction from each of the three formats(R, I, S) and convert them into their corresponding binary values.

Note: For opcode, funct3 or any other field values that you do not know just put “X”

Question - 5:

What is the significance of the funct3 field in case of a Load instruction?

## Question - 6:

if ( A[3] != A[6] or  A[6] > 5){

if (A[3] == 0) {

A[3] = A[3] + 2;

}else{

A[6] = A[6] / 16;

}

}else{

A[6] = A[6] * 8

}

a) Construct the equivalent RISC-V code of the above mentioned C code.

Base addresses of array A and B are in register  X20 and X21. Consider both arrays as double word

b) From your converted codes, pick any instruction from each of the three formats(R, I, SB) and convert them into their corresponding binary values.
Note: For opcode, funct3 or any other field values that you do not know just put “X”